CSC1107 — Operating Systems

Project 15: USB File Transfer Activity Driver

Design Documentation & Project Report

Group 10

Raspberry Pi 4 — Raspbian 64-bit

Submission Date: 10 July 2026

Table of Contents

1. Executive Summary

2. System Architecture Overview

3. Directory Structure & Component Map

4. Kernel Module Design (usb_audit.c)

5. Shared Header Design (usb_tracker.h)

6. User-Space C Application Design (usb_monitor.c)

7. Python Watchdog Daemon (file_tracker.py)

8. Bash Automation Script (run.sh)

9. Advanced Challenge: Mass-Copy Anomaly Detection

10. Build System & Makefile Design

11. Communication Protocol — ioctl Interface

12. Logging & Auditing Strategy

13. Demonstration Instructions

14. Error Handling & Debugging

15. Conclusion & Lessons Learned

1. Executive Summary

This document presents the complete design and implementation of a USB File Transfer Activity Driver — a Linux kernel module that monitors file operations on removable USB storage devices.  The system was developed for the Raspberry Pi 4 running Raspbian 64-bit (Linux kernel 6.x) and addresses three core objectives:

• Block Device Monitoring — detecting USB mass-storage device insertion and removal via the kernel's USB notifier chain.

• File Activity Tracking — recording file create, modify, and delete operations with timestamps, process IDs, and transfer sizes.

• Logging & Auditing — maintaining a persistent circular log buffer accessible from user-space, with all events also emitted via printk() for dmesg visibility.

• Advanced Challenge — automatic mass-copy anomaly detection using a sliding time-window ring buffer, configurable thresholds, and cooldown-based alert deduplication.

The solution comprises three layers: (1) a Linux kernel module (.ko) that registers a character device at /dev/usb_audit, (2) a user-space C application that communicates with the driver via ioctl, and (3) a Python watchdog daemon for alternative real-time filesystem monitoring.  A Bash automation script handles the full build → load → run → cleanup lifecycle.

2. System Architecture Overview

The system follows a classic user-space / kernel-space split architecture with a character device as the communication bridge:

┌─────────────────────────────────────────────────────────────────┐
│                     USER SPACE                                  │
│  ┌──────────────────┐    ┌──────────────────────────────────┐   │
│  │  usb_monitor.c   │    │     file_tracker.py              │   │
│  │  (C application) │    │  (Python watchdog daemon)        │   │
│  │  - Interactive   │    │  - Real-time FS monitoring       │   │
│  │  - Daemon mode   │    │  - Mass-copy detection           │   │
│  └────────┬─────────┘    └──────────────────────────────────┘   │
│           │ ioctl / read / write                                │
│           ▼                                                     │
│  ┌─────────────────────────────────────────────────────────┐    │
│  │              /dev/usb_audit  (character device)         │    │
│  └─────────────────────────────────────────────────────────┘    │
├─────────────────────────────────────────────────────────────────┤
│                     KERNEL SPACE                                │
│  ┌─────────────────────────────────────────────────────────┐    │
│  │                 usb_audit.ko  (LKM)                     │    │
│  │  ┌──────────────┐ ┌──────────────┐ ┌──────────────────┐ │    │
│  │  │ USB Notifier │ │ Circular Log │ │ Anomaly Detector │ │    │
│  │  │  Callback    │ │   Buffer     │ │  (ring buffer)   │ │    │
│  │  └──────────────┘ └──────────────┘ └──────────────────┘ │    │
│  │  ┌──────────────────────────────────────────────────────│    │
│  │  │  printk() → dmesg / kernel ring buffer               │    │
│  │  └──────────────────────────────────────────────────────│    │
│  └─────────────────────────────────────────────────────────┘    │
└─────────────────────────────────────────────────────────────────┘

Key architectural decisions:

• Character device over procfs/sysfs — allows standard POSIX open/read/write/ioctl operations and automatic /dev node creation via device_create().

• Circular ring buffer — fixed 128-entry allocation prevents memory exhaustion; oldest entries are silently overwritten when full.

• Mutex-based synchronisation — a single DEFINE_MUTEX protects all shared state (log buffer, statistics, anomaly ring).

• USB notifier chain — hooks into the kernel's existing USB hotplug infrastructure rather than polling, ensuring zero CPU overhead when no USB events occur.

3. Directory Structure & Component Map

The project is organised into a clean, modular layout separating kernel code, user code, shared headers, scripts, and documentation:

OS-Grp-10/
├── .gitignore                  # Build exclusion configurations
├── README.md                   # Full system documentation
├── Makefile                    # Top-level build orchestration
│
├── include/
│   └── usb_tracker.h           # Shared: ioctl commands, data structs, constants
│
├── src_kernel/                 # Linux Kernel Module (LKM)
│   ├── Makefile                # Kbuild script for module compilation
│   └── usb_audit.c             # Character device driver + USB notifier + anomaly
│
├── src_user/                   # User-space applications
│   ├── usb_monitor.c           # C monitor (interactive + daemon modes)
│   └── file_tracker.py         # Python watchdog daemon (alternative monitor)
│
└── scripts/
    └── run.sh                  # Bash automation: build → insmod → run → rmmod

| Component | Language | Role | Key File |
| --- | --- | --- | --- |
| Kernel Module | C | Character device driver + USB notifier + anomaly engine | src_kernel/usb_audit.c |
| Shared Header | C Header | ioctl protocol, data structures, constants | include/usb_tracker.h |
| User-Space App | C | Interactive menu, daemon dashboard, anomaly config | src_user/usb_monitor.c |
| Python Daemon | Python 3 | watchdog-based FS monitor, sliding-window detector | src_user/file_tracker.py |
| Automation | Bash | Full lifecycle: build, load, run, cleanup | scripts/run.sh |
| Build System | Makefile | Top-level + Kbuild for kernel module | Makefile + src_kernel/Makefile |

4. Kernel Module Design — usb_audit.c

The kernel module is the heart of the system.  It is structured into seven logical sections, each implemented as a separate function or group of functions:

4.1 Module Metadata & Global State

The module begins with standard Linux kernel includes and MODULE_* macros declaring the license (GPL), author, description, and version.  Global state is organised into clearly named static variables:

• usb_audit_devt — dynamically allocated major/minor device number.

• usb_audit_cdev — the kernel cdev structure registered with the VFS.

• usb_audit_class / usb_audit_device — sysfs class and /dev node.

• log_buffer — kmalloc'd array of 128 usb_audit_log_entry_t structs.

• log_head / log_count — circular buffer write index and occupancy.

• stats — aggregate counters (usb_audit_stats_t).

• anomaly_ring[64] — ring of ktime_t timestamps for burst detection.

• anomaly_threshold / anomaly_window_ms — configurable detection params.

• audit_mutex — protects all of the above from concurrent access.

4.2 audit_log_event() — Core Logging Function

Every observable event (USB hotplug, file operation, anomaly alert) flows through this single function.  It acquires audit_mutex, writes one entry at log_head with timestamp from ktime_get_real_ns(), advances the head pointer, updates aggregate statistics via a switch on event_type, and calls anomaly_check_burst() for file events.  This centralisation ensures consistency: all log entries share the same format, all stats are updated atomically, and no code path can bypass the synchronisation lock.

4.3 USB Notifier Callback

usb_audit_notify() is registered with usb_register_notify() during module init.  It filters for USB_CLASS_MASS_STORAGE devices and logs DEVICE_IN / DEVICE_OUT events with vendor/product/serial details.  This design leverages the kernel's existing hotplug infrastructure, requiring zero polling and adding negligible overhead.

4.4 Character Device file_operations

The driver implements five fops:

• open() — logs the accessing PID/comm; always returns 0.

• release() — logs device closure.

• read() — dequeues the oldest log entry from the circular buffer.  Returns 0 (EOF) when buffer is empty.

• write() — accepts event strings in format "<code> <path>".  Parses C/M/D/A codes and calls audit_log_event().  This is the primary testing interface.

• unlocked_ioctl() — dispatches 7 command codes (GET_STATS, CLEAR_LOGS, SET_PATH, GET_LOGS, RESET_STATS, SET_ANOMALY, GET_ANOMALY).

4.5 Module Init & Exit

The init sequence follows the standard LKM pattern with careful error unwinding:

• alloc_chrdev_region() → cdev_init() + cdev_add() → class_create() + device_create() → kmalloc_array() for log buffer → memset stats → usb_register_notify().

• If any step fails, the goto-based unwind path reverses all preceding allocations in the correct order.

• The exit function reverses the sequence: unregister notifier → kfree buffer → device_destroy → class_destroy → cdev_del → unregister_chrdev_region.

5. Shared Header Design — usb_tracker.h

This header is the contract between kernel and user space.  It uses only kernel-compatible types (__u8, __u32, __u64) and the standard Linux ioctl encoding macros (_IO, _IOR, _IOW, _IOWR).

5.1 ioctl Command Encoding

All commands use magic number 'U' (0x55).  The _IO* macros encode direction (read/write/read-write), command number (1-7), and the size of the argument struct — enabling the kernel to validate user-space buffer sizes automatically.

| ioctl Command | Code | Direction | Argument Struct |
| --- | --- | --- | --- |
| USB_AUDIT_GET_STATS | _IOR(1) | Kernel → User | usb_audit_stats_t |
| USB_AUDIT_CLEAR_LOGS | _IO(2) | None | — |
| USB_AUDIT_SET_PATH | _IOW(3) | User → Kernel | char[256] |
| USB_AUDIT_GET_LOGS | _IOWR(4) | Bidirectional | usb_audit_logs_t |
| USB_AUDIT_RESET_STATS | _IO(5) | None | — |
| USB_AUDIT_SET_ANOMALY | _IOW(6) | User → Kernel | usb_audit_anomaly_t |
| USB_AUDIT_GET_ANOMALY | _IOR(7) | Kernel → User | usb_audit_anomaly_t |

5.2 Data Structures

• usb_audit_log_entry_t (264 bytes) — fixed-size log entry with event_type, pid, 64-bit nanosecond timestamp, file_size, and a 256-byte filename buffer.  Explicit padding ensures consistent alignment across 32-bit and 64-bit architectures.

• usb_audit_stats_t (64 bytes) — cumulative counters for bytes written, files created/modified/deleted, device insertions/removals, alerts, and current log count.

• usb_audit_logs_t (~34 KB) — batch retrieval structure holding up to 128 log entries.  The count field serves as both input (requested count) and output (actual count returned).

• usb_audit_anomaly_t (16 bytes) — threshold, window_ms, burst_count, and alert_triggered flag for configuring and querying the anomaly detection engine.

6. User-Space C Application Design — usb_monitor.c

The C application is a single-threaded program with two run modes: interactive (test menu) and daemon (periodic dashboard).  It communicates exclusively through /dev/usb_audit using open(), write(), and ioctl() system calls.

6.1 Interactive Mode

Presents a command menu in a loop.  Each command maps to one or more ioctl calls:

• C/M/D — sends a write() to the kernel with the event code and file path.  After each file event, immediately calls check_anomaly_status() to see if the kernel raised an auto-alert.

• S — calls GET_STATS and GET_ANOMALY, displays both.

• L — calls GET_LOGS with count=10, formats timestamps and event labels into a table.

• T — calls GET_ANOMALY (to read current values), then SET_ANOMALY (to apply user-specified threshold/window).

• R — calls RESET_STATS (which also clears the anomaly ring).

• X — calls CLEAR_LOGS to empty the circular buffer.

• Q — sets keep_running=0 (flag checked by signal-safe volatile).

6.2 Daemon Mode

Clears the screen, prints header + stats + anomaly status + 8 most recent log entries, then sleeps for MENU_REFRESH_SEC (2 s).  Repeats until SIGINT/SIGTERM is received.  The anomaly check on every refresh cycle means kernel-detected bursts are displayed prominently without any user action.

6.3 Signal Handling

SIGINT and SIGTERM are caught by signal_handler(), which sets keep_running = 0.  This is declared volatile sig_atomic_t for safe access from signal context.  The main loop checks this flag on each iteration, ensuring clean file-descriptor closure and exit.

7. Python Watchdog Daemon — file_tracker.py

This script provides an alternative, filesystem-level monitoring approach using the watchdog library.  It operates independently of the kernel module and is useful for validation and comparison.

7.1 Design

• USBMonitorHandler extends FileSystemEventHandler, overriding on_created(), on_modified(), and on_closed().

• Each file event records a timestamp in modification_history[].

• check_anomaly() applies a sliding window filter — events older than TIME_WINDOW (3.0 s) are pruned, and if the remaining count exceeds ALERT_THRESHOLD (5), a [SECURITY ALERT] is printed.

• The Observer runs on a background thread watching USB_TARGET_PATH recursively.

• The main thread blocks on time.sleep(1) in a loop, catching KeyboardInterrupt for clean shutdown.

7.2 Configuration

USB_TARGET_PATH = "/media/pi/SAMSUNG"   # Set via lsblk
ALERT_THRESHOLD = 5                  # Max file ops before alert
TIME_WINDOW     = 3.0                # Sliding window in seconds

8. Bash Automation Script — run.sh

The run.sh script implements an 8-step lifecycle with colour-coded output, error checking at each step, and a trap-based cleanup handler:

| Step | Action | Error Handling |
| --- | --- | --- |
| 1 | Verify root privileges ($EUID == 0) | Exit with message if not root |
| 2 | Check kernel headers exist | Guide user to apt install |
| 3 | make -C src_kernel (build .ko) | Exit on compilation failure |
| 4 | gcc usb_monitor.c (build user app) | Exit on compilation failure |
| 5 | insmod src_kernel/usb_audit.ko | Idempotent: rmmod first if loaded |
| 6 | Verify /dev/usb_audit exists | Print dmesg tail for diagnosis |
| 7 | chmod 666 /dev/usb_audit | Non-fatal if fails |
| 8 | Launch ./usb_monitor & wait | trap ensures rmmod on any exit |

The trap 'cleanup EXIT INT TERM' ensures the kernel module is always unloaded (rmmod) and the user app is killed, even if the user presses Ctrl+C or the script encounters an error.

9. Advanced Challenge: Automatic Mass-Copy Anomaly Detection

The advanced challenge — "Detect suspicious mass-copy behavior and trigger alerts" — is fully implemented across all three layers of the system.  Below is the step-by-step design rationale.

9.1 Problem Analysis

Mass-copy (data exfiltration) is characterised by a burst of file write operations to a removable device within a short time window.  A simple threshold approach ("alert if more than N files are written") is insufficient because legitimate bulk operations (e.g., backups) would trigger false positives.  We need a sliding window that captures temporal locality.

9.2 Kernel-Side Implementation

The kernel-side detector operates inside audit_log_event() — specifically in the FILE_CREATE and FILE_MODIFY branches.  This ensures every file event is evaluated at the point of recording, with audit_mutex already held for atomicity.

Step 1: Timestamp Recording

A static array anomaly_ring[64] of ktime_t stores the nanosecond timestamp of every file event.  anomaly_ring_idx tracks the write position; anomaly_ring_count tracks occupancy (capped at 64).  ktime_get_real() provides a monotonically increasing clock.

Step 2: Sliding Window Count

On each event, the function computes window_start = now - (anomaly_window_ms * 1,000,000) and iterates backwards through the ring from the most recent entry.  Because entries are stored chronologically, the loop can break on the first timestamp older than window_start — achieving O(burst) rather than O(ring_size).

Step 3: Threshold Comparison & Cooldown

If burst > anomaly_threshold AND the time since anomaly_last_alert exceeds USB_AUDIT_ANOMALY_COOLDOWN_MS (5,000 ms), an alert is raised.  The cooldown prevents alert storms — a sustained copy operation generates one alert per 5 seconds rather than one per file.

Step 4: Alert Logging

The alert is written directly into the log buffer (we hold the mutex, so no re-locking needed) as a USB_AUDIT_EVENT_ALERT entry.  A printk(KERN_WARNING) message is emitted for dmesg visibility.  The stats.alert_count counter is incremented.

9.3 User-Space Integration

• The T command in interactive mode configures threshold and window at runtime via USB_AUDIT_SET_ANOMALY.

• check_anomaly_status() queries USB_AUDIT_GET_ANOMALY after every file event in interactive mode and every refresh cycle in daemon mode, displaying a prominent red-bordered warning when alert_triggered == 1.

• CLI options -t/--threshold and -w/--window allow configuring anomaly detection at launch time.

• The Python file_tracker.py provides an independent second opinion using the same sliding-window algorithm at the filesystem level.

9.4 Design Rationale Summary

| Design Decision | Rationale |
| --- | --- |
| Ring buffer in kernel | Zero heap allocation after init; O(1) insert |
| Cooldown period | Prevents alert fatigue during sustained copies |
| Chronological storage | Enables early loop exit for O(burst) scan |
| Configurable at runtime | No recompilation needed to tune sensitivity |
| Dual detection (kernel + Python) | Defence in depth; kernel catches all, Python validates |
| printk + log buffer | Alerts visible in both dmesg and user-space dashboard |

10. Build System & Makefile Design

The project uses two Makefiles in a parent-child relationship:

10.1 Top-Level Makefile

Provides three targets:

• make all — invokes make -C src_kernel for the kernel module and gcc for the user app.

• make kernel — builds only src_kernel/usb_audit.ko.

• make user — compiles src_user/usb_monitor.c with -Wall -Wextra -O2 -I./include.

• make clean — removes .ko, .o, and the usb_monitor binary.

10.2 Kernel Makefile (Kbuild)

Uses the standard obj-m := usb_audit.o pattern.  The KDIR variable resolves to /lib/modules/$(uname -r)/build, ensuring the module is compiled against the exact running kernel's headers.  The M=$(PWD) directive tells Kbuild to look for the source in the current directory.

11. Communication Protocol — ioctl Interface

The ioctl interface is the primary communication channel between user-space and kernel-space.  It was chosen over alternatives (netlink, sysfs, procfs) for several reasons:

• Type safety — the _IO* macros encode argument sizes, allowing the kernel to validate user-space buffers.

• Bidirectional — _IOWR commands pass data in both directions in a single system call.

• Familiar — standard POSIX ioctl semantics; well-understood by kernel and system programmers.

• Extensible — new commands can be added by incrementing USB_AUDIT_MAX_NR and adding a case to the switch.

Data flow for a typical GET_STATS call:

User Space                          Kernel Space
──────────                          ────────────
usb_audit_stats_t st;
ioctl(fd, USB_AUDIT_GET_STATS, &st);
                                     │
                                     ▼
                            usb_audit_ioctl()
                              │
                              ├─ Validate magic number (_IOC_TYPE)
                              ├─ Validate command number (_IOC_NR)
                              ├─ mutex_lock(&audit_mutex)
                              ├─ copy_to_user(ustats, &stats, ...)
                              ├─ mutex_unlock(&audit_mutex)
                              └─ return 0
                                     │
        st now contains            ◄──┘
        aggregate counters

12. Logging & Auditing Strategy

The system implements dual-path logging to satisfy both kernel debugging and user-space auditing requirements:

12.1 Kernel Path — printk() / dmesg

Every significant event produces a printk() message with the "[usb_audit]" prefix for easy grepping.  Log levels are chosen carefully:

• KERN_INFO — normal operations: init, exit, device events, file events.

• KERN_WARNING — anomaly alerts (mass-copy detected).

• KERN_ERR — initialisation failures (allocation, registration).

• KERN_DEBUG — device open/close (low-volume diagnostic).

12.2 User-Space Path — /dev/usb_audit read/write

The circular log buffer stores structured usb_audit_log_entry_t records with nanosecond timestamps.  The user app retrieves them in batches via USB_AUDIT_GET_LOGS, formats timestamps into human-readable HH:MM:SS, and displays them in a colour-coded table.

12.3 Audit Trail Completeness

The combination of dmesg (real-time kernel perspective) and the structured log buffer (queryable from user-space) provides a complete audit trail.  Events are never dropped silently — the circular buffer overwrites oldest-first, and the overwritten count can be inferred from cumulative statistics vs. buffer size.

13. Demonstration Instructions

Follow these steps to demonstrate all required functionality on the Raspberry Pi 4:

13.1 Automated Demo (Recommended)

# Step 1: Make the script executable
chmod +x scripts/run.sh

# Step 2: Run the full automated demo
sudo ./scripts/run.sh --interactive

# The script will:
#   → Build the kernel module
#   → Build the user app
#   → Insert the module
#   → Verify /dev/usb_audit
#   → Launch the interactive monitor
#   → On Ctrl+C: unload module, clean up

13.2 Manual Demo

# Terminal 1: Build and load
make all
sudo insmod src_kernel/usb_audit.ko
ls -la /dev/usb_audit        # verify device node
dmesg | tail -10             # verify kernel logs

# Terminal 1: Run the monitor
sudo ./src_user/usb_monitor --interactive

# In the interactive menu, demonstrate:
C test1.txt   # File create
C test2.txt   # Another create
M test1.txt   # File modify
S             # Show stats
L             # Show logs

# Advanced challenge: trigger mass-copy alert
# Send 6 rapid CREATE events:
C file1.txt
C file2.txt
C file3.txt
C file4.txt
C file5.txt
C file6.txt   # ← ALERT fires automatically!

# Verify in dmesg:
dmesg | grep "SECURITY ALERT"

# Terminal 2: Run Python watchdog
python3 src_user/file_tracker.py
echo "test" > "/media/pi/SAMSUNG/demo.txt"

# Cleanup:
sudo rmmod usb_audit
make clean

13.3 Demo Checklist

| # | Requirement | How to Verify |
| --- | --- | --- |
| 1 | Module compilation | make kernel — zero errors |
| 2 | Module insertion | sudo insmod → lsmod ¦ grep usb_audit |
| 3 | Module removal | sudo rmmod → lsmod shows empty |
| 4 | Hardware detection | Plug/unplug USB → dmesg shows INSERTED/REMOVED |
| 5 | App ↔ driver comm | Interactive menu C/M/D → driver responds |
| 6 | dmesg logging | dmesg ¦ grep usb_audit shows all events |
| 7 | User app interaction | All menu commands functional |
| 8 | Clean shutdown | Ctrl+C in run.sh → rmmod called |
| 9 | Transfer statistics | S command shows aggregate counters |
| 10 | Transfer logs | L command shows formatted log table |
| 11 | Mass-copy alert | 6 rapid creates → auto ALERT in dmesg + dashboard |

14. Error Handling & Debugging Strategy

14.1 Kernel Module Error Handling

The init function uses a goto-based unwind pattern.  Each allocation step checks the return value; on failure, control jumps to the appropriate cleanup label which reverses all preceding allocations in the correct order.  This ensures the module either loads completely or cleans up cleanly — never leaving a partially initialised state.

• alloc_chrdev_region failure → return immediately (nothing to undo).

• cdev_add failure → goto err_unregister_region.

• class_create failure → goto err_cdev_del.

• device_create failure → goto err_class_destroy.

• kmalloc failure → goto err_device_destroy (ENOMEM).

14.2 User-Space Error Handling

• All ioctl calls are wrapped in if-checks with perror() on failure.

• open() failure prints a diagnostic suggesting insmod.

• Signal handlers use volatile sig_atomic_t for safe flag setting.

• fgets() return value is checked to detect EOF on stdin.

14.3 Debugging Tools

# View kernel messages in real-time
dmesg -w

# Check module is loaded
lsmod | grep usb_audit

# Check device node permissions
ls -la /dev/usb_audit

# Verify module info
modinfo src_kernel/usb_audit.ko

# Test ioctl from command line (if available)
./src_user/usb_monitor -h

15. Conclusion & Lessons Learned

This project successfully implements a complete Linux kernel driver for USB file transfer monitoring, meeting all core requirements and the advanced challenge goal.

15.1 Key Achievements

• Full-featured character device driver with dynamic major number allocation and automatic /dev node creation.

• Seven ioctl commands covering statistics, logs, configuration, and anomaly detection.

• USB hotplug monitoring via the kernel notifier chain, requiring zero polling overhead.

• Circular ring buffer for bounded-memory, O(1) event logging.

• Automatic mass-copy anomaly detection with configurable threshold, sliding window, and cooldown-based deduplication.

• Dual user-space interfaces: C application (ioctl-based) and Python daemon (watchdog-based) for defence in depth.

• Bash automation script handling the full lifecycle with trap-based cleanup guarantees.

• Comprehensive printk() logging at appropriate severity levels.

15.2 Design Principles Applied

• Separation of concerns — kernel handles detection; user-space handles display and configuration.

• Defence in depth — kernel-side anomaly detection + Python watchdog provide independent verification.

• Least astonishment — standard POSIX interfaces (open/read/write/ioctl), standard Kbuild, standard LKM patterns.

• Fail-safe — every initialisation step has a corresponding cleanup step; trap ensures rmmod on any exit path.

• Observability — every event logged in both dmesg and structured buffer; stats always queryable.

15.3 Lessons Learned

• Kernel development requires careful attention to concurrency — a single mutex simplified the design considerably while maintaining correctness.

• The ioctl command encoding macros (_IO, _IOR, etc.) are essential for type-safe user/kernel communication and should never be bypassed with raw integer commands.

• Circular buffers are ideal for logging in constrained environments but require careful handling of the full-vs-empty ambiguity (solved by tracking occupancy separately).

• Cooldown mechanisms in anomaly detection are critical for usability — without them, a sustained copy generates one alert per file, drowning operators in noise.

— End of Report —